Filtering and Navigation

TD1

Luenberger Observer Design of a 2nd Order LTI System

Objectives:

- Design a Luenberger observer for a 2nd order LTI system with given parameters

- Understand the behavior of the observer on different conditions

Tasks:

- Construct a state space representation of a 2nd order LTI system with given  and 

- Simulate the response of the system with different input (a constant input, a square wave and a sine). Change the natural frequency  and damping factor to see how the system will react. 

- The system is measured by certain sensor, write the output equation.

- Verify the system is observable.

- Synthesize a Luenberger observer to observe the system. Find out the observer gain L with the required  and and given  and of the system.

- In the real world, the sensor must be corrupted by some noise. Here we suppose it is white, plot the response of the observer.

- Add a model error to the observer and simulate the response.

Fig. 1 System & observer response with noisy input

Fig.2 Observer with system model error, the real system  and  while with the system error  and 